+-----------+------------+-------------+-------------+-------------+-------------+-------------------+

| Component | Technology | Amnt of ROM | Type of ROM | Amnt of RAM | No. of SFRs |  Serial I/O Type  |

|-----------|------------|-------------|-------------|-------------|-------------|-------------------|

|    8031   |    HMOS    |      0      |     --      |  128 bytes  |     21      | Start/Stop Async  |

|    8051   |    HMOS    |   4Kbytes   |   Masked    |  128 bytes  |     21      | Start/Stop Async  |

|    8751   |    HMOS    |   4Kbytes   |   EPROM     |  128 bytes  |     21      | Start/Stop Async  |

|    8053   |    HMOS    |   8Kbytes   |   Masked    |  128 bytes  |     21      | Start/Stop Async  |

|    9761   |    HMOS    |   8Kbytes   |   EPROM     |  128 bytes  |     21      | Start/Stop Async  |

|    8751   |    HMOS    |   8Kbytes   |   EPROM     |  128 bytes  |     21      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|   80C31   |    CMOS    |      0      |     --      |  128 bytes  |     21      | Start/Stop Async  |

|   80C51   |    CMOS    |   4Kbytes   |   Masked    |  128 bytes  |     21      | Start/Stop Async  |

|   87C51   |    CMOS    |   4Kbytes   |   EPROM     |  128 bytes  |     21      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|    8032   |    HMOS    |      0      |     --      |  256 bytes  |     26      | Start/Stop Async  |

|    8052   |    HMOS    |   8Kbytes   |   Masked    |  256 bytes  |     26      | Start/Stop Async  |

|    8752   |    HMOS    |   8Kbytes   |   EPROM     |  256 bytes  |     26      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|   80C32   |    CMOS    |      0      |     --      |  256 bytes  |     26      | Start/Stop Async  |

|   80C52   |    CMOS    |   8Kbytes   |   Masked    |  256 bytes  |     26      | Start/Stop Async  |

|   87C52   |    CMOS    |   8Kbytes   |   EPROM     |  256 bytes  |     26      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|    8044   |    HMOS    |   4Kbytes   |   Masked    |  192 bytes  |     34      |     HDLC/SDLC     |

|    8344   |    HMOS    |      0      |     --      |  192 bytes  |     34      |     HDLC/SDLC     |

|    8744   |    HMOS    |   4Kbytes   |   EPROM     |  192 bytes  |     34      |     HDLC/SDLC     |

|           |            |             |             |             |             |                   |

|   80535   |    HMOS    |      0      |     --      |  256 bytes  |     41      | Start/Stop Async  |

|   80515   |    HMOS    |   8Kbytes   |   Masked    |  256 bytes  |     41      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C535   |   CHMOS    |      0      |     --      |  256 bytes  |     41      | Start/Stop Async  |

|  80C515   |   CHMOS    |   8Kbytes   |   Masked    |  256 bytes  |     41      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|   80532   |    HMOS    |      0      |     --      |  128 bytes  |     28      | Start/Stop Async  |

|   80512   |    HMOS    |   4Kbytes   |   Masked    |  128 bytes  |     28      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C152   |   CHMOS    |      0      |     --      |  256 bytes  |     56      |      CSMA/CD      |

|  83C152   |   CHMOS    |   8Kbytes   |   Masked    |  256 bytes  |     56      |      CSMA/CD      |

|           |            |             |             |             |             |                   |

|  80C154   |    CMOS    |      0      |     --      |  256 bytes  |     27      | Start/Stop Async  |

|  83C154   |    CMOS    |  16Kbytes   |   Masked    |  256 bytes  |     27      | Start/Stop Async  |

|  85C154   |    CMOS    |  16Kbytes   |   EPROM     |  256 bytes  |     27      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

+-----------+------------+-------------+-------------+-------------+-------------+-------------------+

+-----------+------------+-------------+-------------+-------------+-------------+-------------------+

| Component | Technology | Amnt of ROM | Type of ROM | Amnt of RAM | No. of SFRs |  Serial I/O Type  |

|-----------|------------|-------------|-------------|-------------|-------------|-------------------|

|  80C51FA  |   CHMOS    |      0      |     --      |  256 bytes  |     47      | Start/Stop Async  |

|  83C51FA  |   CHMOS    |   8Kbytes   |   Masked    |  256 bytes  |     47      | Start/Stop Async  |

|  87C51FA  |   CHMOS    |   8Kbytes   |   EPROM     |  256 bytes  |     47      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  83C51FB  |   CHMOS    |  16Kbytes   |   Masked    |  256 bytes  |     47      | Start/Stop Async  |

|  87C51FB  |   CHMOS    |  16Kbytes   |   EPROM     |  256 bytes  |     47      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  83C51FB  |   CHMOS    |  32Kbytes   |   Masked    |  256 bytes  |     47      | Start/Stop Async  |

|  87C51FB  |   CHMOS    |  32Kbytes   |   EPROM     |  256 bytes  |     47      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C537   |   CHMOS    |      0      |     --      |  256 bytes  |     41      | Start/Stop Async  |

|  80C517   |   CHMOS    |   8Kbytes   |   Masked    |  256 bytes  |     82      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C451   |   CMOS     |      0      |     --      |  128 bytes  |     24      |    Parallel I/F   |

|  83C451   |   CMOS     |   4Kbytes   |   Masked    |  128 bytes  |     24      |    Parallel I/F   |

|  87C451   |   CMOS     |   4Kbytes   |   EPROM     |  128 bytes  |     24      |    Parallel I/F   |

|           |            |             |             |             |             |                   |

|  80C452   |   CHMOS    |      0      |     --      |  256 bytes  |     55      |      U.P.I.       |

|  83C452   |   CHMOS    |   8Kbytes   |     --      |  256 bytes  |     55      |      U.P.I.       |

|  87C452   |   CHMOS    |   8Kbytes   |     --      |  256 bytes  |     55      |      U.P.I.       |

|           |            |             |             |             |             |                   |

|  80C552   |   CMOS     |      0      |     --      |  256 bytes  |     54      | Start/Stop Async  |

|  83C552   |   CMOS     |   8Kbytes   |   Masked    |  256 bytes  |     54      | Start/Stop Async  |

|  87C552   |   CMOS     |   8Kbytes   |   EPROM     |  256  bytes |     54      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C652   |   CMOS     |      0      |     --      |  256 bytes  |     24      | Start/Stop Async  |

|  83C652   |   CMOS     |   8Kbytes   |   Masked    |  256 bytes  |     24      | Start/Stop Async  |

|  87C652   |   CMOS     |   8Kbytes   |   EPROM     |  256 bytes  |     24      | Start/Stop Async  |

|  83C654   |   CMOS     |  16Kbytes   |   Masked    |  256 bytes  |     24      | Start/Stop Async  |

|  87C654   |   CMOS     |  16Kbytes   |   EPROM     |  256 bytes  |     24      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  83C752   |   CMOS     |   2Kbytes   |   Masked    |   64 bytes  |     25      |      I2C          |

|  87C752   |   CMOS     |   2Kbytes   |   EPROM     |   64 bytes  |     25      |      I2C          |

|           |            |             |             |             |             |                   |

|  83C751   |   CMOS     |   2Kbytes   |   Masked    |   64 bytes  |     20      |      I2C          |

|  87C751   |   CMOS     |   2Kbytes   |   EPROM     |   64 bytes  |     20      |      I2C          |

|           |            |             |             |             |             |                   |

|  80C521   |   CMOS     |      0      |     --      |  256 bytes  |     26      | Start/Stop Async  |

|  80C321   |   CMOS     |   8Kbytes   |   Masked    |  256 bytes  |     26      | Start/Stop Async  |

|  87C521   |   CMOS     |   8Kbytes   |   EPROM     |  256 bytes  |     26      | Start/Stop Async  |

|  80C541   |   CMOS     |  16Kbytes   |   Masked    |  256 bytes  |     26      | Start/Stop Async  |

|  87C541   |   CMOS     |  16Kbytes   |   EPROM     |  256 bytes  |     26      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

|  80C851   |    CMOS    |      0      |     --      |  128 bytes  |     21      | Start/Stop Async  |

|  83C851   |    CMOS    |   4Kbytes   |   Masked    |  128 bytes  |     21      | Start/Stop Async  |

|           |            |             |             |             |             |                   |

+-----------+------------+-------------+-------------+-------------+-------------+-------------------+

$DATE(date)       Places date in page header               $EJECT            Places a form feed in listing

$INCLUDE(file)    Inserts file in source program           $LIST             Allows listing to be output

$NOLIST           Stops outputting the listing             $MOD51            Uses 8051 predefined symbols

$MOD52            Uses 8052 predefined symbols             $MOD44            Uses 8044 predefined symbols

$NOMOD            No predefined symbols used               $OBJECT(file)     Places object output in file

$NOOBJECT         No object file is generated              $PAGING           Break output listing into pages

$NOPAGING         Print listing w/o page breaks            $PAGELENGTH(n)    No. of lines on a listing page

$PAGEWIDTH(n)     No. of columns on a listing page         $PRINT(file)      Places listing output in file

$NOPRINT          Listing will not be output               $SYMBOLS          Append symbol table to listing

$NOSYMBOLS        Symbol table will not be output          $TITLE(string)    Places string in page header

EQU       Define symbol                                    DATA      Define internal memory symbol

IDATA     Define indirectly addressed internal             XDATA     Define external memory symbol

memory symbol                                BIT       Define internal bit memory symbol

CODE      Define program memory symbol                     DS        Reserve bytes of data memory

DBIT      Reserve bits of bit memory                       DB        Store byte values in program memory

DW        Store word values in program memory              ORG       Set segment location counter

END       End of assembly language source file             CSEG      Select program memory space

DSEG      Select internal memory data space                XSEG      Select external memory data space

ISEG      Select indirectly addressed internal             BSEG      Select bit addressable memory space

memory space                                 USING     Select register bank

IF        Begin conditional assembly block                 ELSE      Alternative conditional assembly block

ENDIF     End conditional assembly block

ACALL     Absolute call            ADD       Add                       ADDC      Add with carry

AJMP      Absolute jump            ANL       Logical and               CJNE      Compare & jump if not equal

CLR       Clear                    CPL       Complement                DA        Decimal adjust

DEC       Decrement                DIV       Divide                    DJNZ      Decrement & jump if not zero

INC       Increment                JB        Jump if bit set           JBC       Jump & clear bit if bit set

JC        Jump if carry set        JMP       Jump                      JNB       Jump if bit not set

JNC       Jump if carry not set    JNZ       Jump if accum. not zero   JZ        Jump if accumulator zero

LCALL     Long call                LJMP      Long jump                 MOV       Move

MOVC      Move code                MOVX      Move external             MUL       Multiply

NOP       No operation             ORL       Inclusive or              POP       Pop stack

PUSH      Push stack               RET       Return                    RETI      Return from interrupt

RL        Rotate left              RLC       Rotate left thru carry    RR        Rotate right

RRC       Rotate right thru carry  SETB      Set bit                   SJMP      Short jump

SUBB      Subtract with borrow     SWAP      Swap nibbles              XCH       Exchange bytes

XCHD      Exchange digits          XRL       Exclusive or              CALL      Generic call

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |    FORM                |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ACALL                 |                         |             |             |                        |       |   |   |       |

|  2K in Page (11 bits)  | PC + 2 -> STACK         | Prog Direct |             | ACALL  Paddr           |  see  | 2 | 2 | - - - |

|  Absolute Call         | SP + 2 -> SP            |             |             |                        | note  |   |   |       |

|                        | Paddr<10:0> -> PC<10:0> |             |             |                        |   1   |   |   |       |

|                        | PC<15:11> -> PC<15:11>  |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ADD                   |                         |             |             |                        |       |   |   |       |

|  Add Operand to        | A + SOper -> A          | Accumulator | Immediate   | ADD   A,#data          |  24   | 2 | 1 | * * * |

|  Accumulator           |                         |      "      | Direct      | ADD   A,Daddr          |  25   | 2 | 1 |       |

|                        |                         |      "      | Indirect    | ADD   A,@Ri            | 26,27 | 1 | 1 |       |

|                        |                         |      "      | Register    | ADD   A,Rn             | 28-2F | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ADDC                  |                         |             |             |                        |       |   |   |       |

|  Add Operand with      | A + SOper + C -> A      | Accumulator | Immediate   | ADDC  A,#data          |  34   | 2 | 1 | * * * |

|  Carry to Accumulator  |                         |      "      | Direct      | ADDC  A,Daddr          |  35   | 2 | 1 |       |

|                        |                         |      "      | Indirect    | ADDC  A,@Ri            | 36,37 | 1 | 1 |       |

|                        |                         |      "      | Register    | ADDC  A,Rn             | 38-3F | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  AJMP                  |                         |             |             |                        |  see  |   |   |       |

|  2K in Page (11 bits)  | Paddr<10:0> -> PC<10:0> | Prog Direct |             | AJMP  Paddr            | note  | 2 | 2 | - - - |

|  Absolute Jump         | PC<15:11> -> PC<15:11>  |             |             |                        |   2   |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ANL                   |                         |             |             |                        |       |   |   |       |

|  Logical AND of Source | SOper ^ DOper -> DOper  | Direct      | Accumulator | ANL   Daddr,A          |  52   | 2 | 1 | - - - |

|  Operand with          |                         |   "         | Immediate   | ANL   Daddr,#data      |  53   | 3 | 2 |       |

|  Destination Operand   |                         | Accumulator | Immediate   | ANL   A,#data          |  54   | 2 | 1 |       |

|                        |                         |      "      | Direct      | ANL   A,Daddr          |  55   | 2 | 1 |       |

|                        |                         |      "      | Indirect    | ANL   A,@Ri            | 56,57 | 1 | 1 |       |

|                        |                         |      "      | Register    | ANL   A,Rn             | 58-5F | 1 | 1 |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Logical AND of Source | SOper ^ CF -> CF        | Carry Flag  | Bit Direct  | ANL   C,Baddr          |  82   | 2 | 2 | * - - |

|  Operand with          |                         |             |             |                        |       |   |   |       |

|  Carry Flag (Continued)|                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ANL (Continued)       |                         |             |             |                        |       |   |   |       |

|  Logical AND of Source | ~SOper ^ CF -> CF       | Carry Flag  | Bit Direct  | ANL   C,/Baddr         |  B0   | 2 | 2 | * - - |

|  Operand Complemented  |                         |             |             |                        |       |   |   |       |

|  with Carry Flag       |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  CJNE                  |                         |             |             |                        |       |   |   |       |

|  Compare Operands and  | Jump Relative to PC if  | Accumulator | Immediate   | CJNE  A,#data,Roff     |  B4   | 3 | 2 | * - - |

|  Jump Relative if not  | DOper <> SOper          |      "      | Direct      | CJNE  A,Daddr,Roff     |  B5   | 3 | 2 |  see  |

|  Equal                 |                         | Indirect    | Immediate   | CJNE  @Ri,#data,Roff   | B6,B7 | 3 | 2 | note  |

|                        |                         | Register    |     "       | CJNE  Rn,#data,Roff    | B8-BF | 3 | 2 |   3   |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  CLR                   |                         |             |             |                        |       |   |   |       |

|  Clear Accumulator     | 0 -> A                  | Accumulator |             | CLR   A                |  E4   | 1 | 1 | - - - |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Clear Carry Flag      | 0 -> CF                 | Carry Flag  |             | CLR   C                |  C3   | 1 | 1 | 0 - - |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Clear Bit Operand     | 0 -> DOper              | Bit Direct  |             | CLR   Baddr            |  C2   | 2 | 1 | - - - |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  CPL                   |                         |             |             |                        |       |   |   |       |

|  Complement Accumulator| ~A -> A                 | Accumulator |             | CPL   A                |  F4   | 1 | 1 | - - - |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Complement Carry Flag | ~CF -> CF               | Carry Flag  |             | CPL   C                |  B3   | 1 | 1 | * - - |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Complement Bit Operand| ~DOper -> DOper         | Bit Direct  |             | CPL   Baddr            |  B2   | 2 | 1 | - - - |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  DA                    |                         |             |             |                        |       |   |   |       |

|  Decimal Adjust        | If (A<3:0> > 9) v AC    | Accumulator |             | DA    A                |  D4   | 1 | 1 | * - - |

|  Accumulator for       |  then A<3:0>+6 -> A<3:0>|             |             |                        |       |   |   |  see  |

|  Addition              | If (A<7:4> > 9) v CF    |             |             |                        |       |   |   | note  |

|                        |  then A<7:4>+6 -> A<7:4>|             |             |                        |       |   |   |   4   |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  DEC                   |                         |             |             |                        |       |   |   |       |

|  Decrement Operand     | DOper - 1 -> DOper      | Accumulator |             | DEC   A                |  14   | 1 | 1 | - - - |

|                        |                         | Direct      |             | DEC   Daddr            |  15   | 2 | 1 |       |

|                        |                         | Indirect    |             | DEC   @Ri              | 16,17 | 1 | 1 |       |

|                        |                         | Register    |             | DEC   Rn               | 18-1F | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  DIV                   |                         |             |             |                        |       |   |   |       |

|  Divide Accumulator by | A / B -> A              |  AB         |             | DIV   AB               |  84   | 1 | 4 | 0 - * |

|  B Register            | Remainder -> B          |             |             |                        |       |   |   |  see  |

|                        |                         |             |             |                        |       |   |   | note 5|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  DJNZ                  |                         |             |             |                        |       |   |   |       |

|  Decrement Operand and | DOper - 1 -> DOper      | Direct      |             | DJNZ  Daddr,Roff       |  D5   | 3 | 2 | - - - |

|  Jump Relative if Not  | If DOper <> 0 then Jump | Register    |             | DJNZ  Rn,Roff          | D8-DF | 2 | 2 |       |

|  Zero                  |  Relative to PC         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  INC                   |                         |             |             |                        |       |   |   |       |

|  Increment Operand     | DOper + 1 -> DOper      | Accumulator |             | INC   A                |  04   | 1 | 1 | - - - |

|                        |                         | Direct      |             | INC   Daddr            |  05   | 2 | 1 |       |

|                        |                         | Indirect    |             | INC   @Ri              | 06,07 | 1 | 1 |       |

|                        |                         | Register    |             | INC   Rn               | 08-0F | 1 | 1 |       |

|                        |                         | Data Pointer|             | INC   DPTR             |  A3   | 1 | 2 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JB                    |                         |             |             |                        |       |   |   |       |

|  Jump Relative if Bit  | If DOper = 1 then Jump  | Bit Direct  |             | JB    Baddr,Roff       |  20   | 3 | 2 | - - - |

|  Operand is Set        |  Relative to PC         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JBC                   |                         |             |             |                        |       |   |   |       |

|  Jump Relative if Bit  | If DOper = 1 then       | Bit Direct  |             | JBC   Baddr,Roff       |  10   | 3 | 2 | * * * |

|  Operand is Set and    |  0 -> DOper and Jump    |             |             |                        |       |   |   |  see  |

|  Clear Bit Operand     |  Relative to PC         |             |             |                        |       |   |   | note 6|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JC                    |                         |             |             |                        |       |   |   |       |

|  Jump Relative if      | If CF = 1 then Jump     | Carry Flag  |             | JC    Roff             |  40   | 2 | 2 | - - - |

|  Carry Flag is Set     |  Relative to PC         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JMP                   |                         |             |             |                        |       |   |   |       |

|  Jump Indirect         | DPTR<15:0> + A<7:0>     | Prog Indir  |             | JMP   @A+DPTR          |  73   | 1 | 2 | - - - |

|                        |   -> PC<15:0>           |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JNB                   |                         |             |             |                        |       |   |   |       |

|  Jump Relative if Bit  | If DOper = 0 then Jump  | Bit Direct  |             | JNB   Baddr,Roff       |  30   | 3 | 2 | - - - |

|  Operand is Clear      |  Relative to PC         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JNC                   |                         |             |             |                        |       |   |   |       |

|  Jump Relative if      | If CF = 0 then Jump     | Carry Flag  |             | JNC   Roff             |  50   | 2 | 2 | - - - |

|  Carry Flag is Clear   |  Relative to PC         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JNZ                   |                         |             |             |                        |       |   |   |       |

|  Jump Relative if the  | If A<7:0> <> 0 then     | Accumulator |             | JNZ   Roff             |  70   | 2 | 2 | - - - |

|  Accumulator is Not    |  Jump Relative to PC    |             |             |                        |       |   |   |       |

|  Zero                  |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  JZ                    |                         |             |             |                        |       |   |   |       |

|  Jump Relative if the  | If A<7:0> = 0 then      | Accumulator |             | JZ    Roff             |  60   | 2 | 2 | - - - |

|  Accumulator is Zero   |  Jump Relative to PC    |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  LCALL                 |                         |             |             |                        |       |   |   |       |

|  Long (16 bits) Call   | PC + 3 -> STACK         | Prog Direct |             | LCALL Paddr            |  12   | 3 | 2 | - - - |

|                        | SP + 2 -> SP            |             |             |                        |       |   |   |       |

|                        | Paddr<15:0> -> PC<15:0> |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  LJMP                  |                         |             |             |                        |       |   |   |       |

|  Long (16 bits)        | Paddr<15:0> -> PC<15:0> | Prog Direct |             | LJMP  Paddr            |  02   | 3 | 2 | - - - |

|  Absolute Jump         |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  MOV                   |                         |             |             |                        |       |   |   |       |

|  Move Source Operand   | SOper -> DOper          | Accumulator | Immediate   | MOV   A,#data          |  74   | 2 | 1 | - - - |

|  to Destination        |                         |      "      | Direct      | MOV   A,Daddr          |  E5   | 2 | 1 |       |

|  Operand               |                         |      "      | Indirect    | MOV   A,@Ri            | E6,E7 | 1 | 1 |       |

|                        |                         |      "      | Register    | MOV   A,Rn             | E8-EF | 1 | 1 |       |

|                        |                         | Direct      | Accumulator | MOV   Daddr,A          |  F5   | 2 | 1 |       |

|                        |                         |   "         | Immediate   | MOV   Daddr,#data      |  75   | 3 | 2 |       |

|                        |                         |   "         | Direct      | MOV   Daddr,Daddr      |  85   | 3 | 2 |       |

|                        |                         |   "         | Indirect    | MOV   Daddr,@Ri        | 86,87 | 2 | 2 |       |

|                        |                         |   "         | Register    | MOV   Daddr,Rn         | 88-8F | 2 | 2 |       |

|                        |                         | Indirect    | Accumulator | MOV   @Ri,A            | F6,F7 | 1 | 1 |       |

|                        | SOper -> DOper          |    "        | Immediate   | MOV   @Ri,#data        | 76,77 | 2 | 1 |       |

|                        |                         |    "        | Direct      | MOV   @Ri,Daddr        | A6,A7 | 2 | 2 |       |

|                        |                         | Register    | Accumulator | MOV   Rn,A             | F8-FF | 1 | 1 |       |

|                        |                         |    "        | Immediate   | MOV   Rn,#data         | 78-7F | 2 | 1 |       |

|                        |                         |    "        | Direct      | MOV   Rn,Daddr         | A8-AF | 2 | 2 |       |

|                        |                         | Data Pointer| Immediate   | MOV   DPTR,#data16     |  90   | 3 | 2 |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Move Carry Flag to Bit| CF -> DOper             | Bit Direct  | Carry Flag  | MOV   Baddr,C          |  92   | 2 | 2 | - - - |

|  Destination Operand   |                         |             |             |                        |       |   |   |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Move Bit Destination  | DOper -> CF             | Carry Flag  | Bit Direct  | MOV   C,Baddr          |  A2   | 2 | 1 | * - - |

|  Operand to Carry Flag |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  MOVC                  |                         |             |             |                        |       |   |   |       |

|  Move byte from        | PM(DPTR<15:0> + A<7:0>) | Accumulator | Prog Ind    | MOVC  A,@A+DPTR        |  93   | 1 | 2 | - - - |

|  Program Memory to     |   -> A<7:0>             |             |             |                        |       |   |   |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        | PM(PC<15:0> + A<7:0>)   | Accumulator | Prog Ind    | MOVC  A,@A+PC          |  83   | 1 | 2 | - - - |

|                        |   -> A<7:0>             |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  MOVX                  |                         |             |             |                        |       |   |   |       |

|  Move byte from        | SOper -> A              | Accumulator | Indirect    | MOVX  A,@Ri            | E2,E3 | 1 | 2 | - - - |

|  External Data Memory  |                         |      "      |    "        | MOVX  A,@DPTR          |  E0   | 1 | 2 |       |

|  to the Accumulator    |                         |             |             |                        |       |   |   |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Move byte in the      | A -> DOper              | Indirect    | Accumulator | MOVX  @Ri,A            | F2,F3 | 1 | 2 | - - - |

|  Accumulator to        |                         |    "        |      "      | MOVX  @DPTR,A          |  F0   | 1 | 2 |       |

|  External Data Memory  |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  MUL                   |                         |             |             |                        |       |   |   |       |

|  Multiply Accumulator  | A X B -> B,A            | AB          |             | MUL   AB               |  A4   | 1 | 4 | 0 - * |

|  by B Register         | (see note 7)            |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  NOP                   |                         |             |             |                        |       |   |   |       |

|  No Operation          |                         |             |             | NOP                    |  00   | 1 | 1 | - - - |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ORL                   |                         |             |             |                        |       |   |   |       |

|  Logical Inclusive OR  | SOper v DOper -> DOper  | Direct      | Accumulator | ORL   Daddr,A          |  42   | 2 | 1 | - - - |

|  of Source Operand     |                         |   "         | Immediate   | ORL   Daddr,#data      |  43   | 3 | 2 |       |

|  with Destination      |                         | Accumulator | Immediate   | ORL   A,#data          |  44   | 2 | 1 |       |

|  Operand               |                         |      "      | Direct      | ORL   A,Daddr          |  45   | 2 | 1 |       |

|                        |                         |      "      | Indirect    | ORL   A,@Ri            | 46,47 | 1 | 1 |       |

|                        |                         |      "      | Register    | ORL   A,Rn             | 48-4F | 1 | 1 |       |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  Logical Inclusive OR  | SOper v CF -> CF        | Carry Flag  | Bit Direct  | ORL   C,Baddr          |  72   | 2 | 2 | * - - |

|  of Source Operand     |                         |             |             |                        |       |   |   |       |

|  with Carry Flag       |                         |             |             |                        |       |   |   |       |

|      (continued)       |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  ORL(continued)        |                         |             |             |                        |       |   |   |       |

|  Logical Inclusive OR  | ~SOper v CF -> CF       | Carry Flag  | Bit Direct  | ORL   C,/Baddr         |  A0   | 2 | 2 | * - - |

|  of Source Operand     |                         |             |             |                        |       |   |   |       |

|  Complemented with     |                         |             |             |                        |       |   |   |       |

|  Carry Flag            |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  POP                   |                         |             |             |                        |       |   |   |       |

|  Pop Stack and Place   | STACK -> DOper          | Direct      | Stack       | POP   Daddr            |  D0   | 2 | 2 | - - - |

|  in Destination Operand| SP - 1 -> SP            |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  PUSH                  |                         |             |             |                        |       |   |   |       |

|  Push Source Operand   | SP + 1 -> SP            | Stack       | Direct      | PUSH  Daddr            |  C0   | 2 | 2 | - - - |

|  onto Stack            | SOper -> STACK          |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RET                   |                         |             |             |                        |       |   |   |       |

|  Return from           | STACK -> PC<15:8>       |             |             | RET                    |  22   | 1 | 2 | - - - |

|  Subroutine            | SP - 1 -> SP            |             |             |                        |       |   |   |       |

|                        | STACK -> PC<7:0>        |             |             |                        |       |   |   |       |

|                        | SP - 1 -> SP            |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RETI                  |                         |             |             |                        |       |   |   |       |

|  Return from           | STACK -> PC<15:8>       |             |             | RETI                   |  32   | 1 | 2 | - - - |

|  Interrupt Routine     | SP - 1 -> SP            |             |             |                        |       |   |   |       |

|                        | STACK -> PC<7:0>        |             |             |                        |       |   |   |       |

|                        | SP - 1 -> SP            |             |             |                        |       |   |   |       |

|                        | 0 -> Intrupt Active Flag|             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RL                    |                         |             |             |                        |       |   |   |       |

|  Rotate Accumulator    | A<6:0> -> A<7:1>        | Accumulator |             | RL    A                |  23   | 1 | 1 | - - - |

|  Left One Bit          | A<7> -> A<0>            |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RLC                   |                         |             |             |                        |       |   |   |       |

|  Rotate Accumulator    | A<6:0> -> A<7:1>        | Accumulator |             | RLC   A                |  33   | 1 | 1 | * - - |

|  Left One Bit Thru     | CF -> A<0>              |             |             |                        |       |   |   |       |

|  the Carry Flag        | A<7> -> CF              |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RR                    |                         |             |             |                        |       |   |   |       |

|  Rotate Accumulator    | A<7:1> -> A<6:0>        | Accumulator |             | RR    A                |  03   | 1 | 1 | - - - |

|  Right One Bit         | A<0> -> A<7>            |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  RRC                   |                         |             |             |                        |       |   |   |       |

|  Rotate Accumulator    | A<7:1> -> A<6:0>        | Accumulator |             | RRC   A                |  13   | 1 | 1 | * - - |

|  Right One Bit Thru    | CF -> A<7>              |             |             |                        |       |   |   |       |

|  the Carry Flag        | A<0> -> CF              |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  SETB                  |                         |             |             |                        |       |   |   |       |

|  Set Bit Operand       | 1 -> CF                 | Carry Flag  |             | SETB  C                |  D3   | 1 | 1 | 1 - - |

|                        |-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        | 1 -> DOper              | Bit Direct  |             | SETB  Baddr            |  D2   | 2 | 1 | - - - |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  SJMP                  |                         |             |             |                        |       |   |   |       |

|  Short (8 bits)        | Jump Relative to PC     |             |             | SJMP  Roff             |  80   | 2 | 2 | - - - |

|  Relative Jump         |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  SUBB                  |                         |             |             |                        |       |   |   |       |

|  Subtract Operand with | A - SOper - CF -> A     | Accumulator | Immediate   | SUBB  A,#data          |  94   | 2 | 1 | * * * |

|  Borrow from the       |                         |      "      | Direct      | SUBB  A,Daddr          |  95   | 2 | 1 |       |

|  Accumulator           |                         |      "      | Indirect    | SUBB  A,@Ri            | 96,97 | 1 | 1 |       |

|                        |                         |      "      | Register    | SUBB  A,Rn             | 98-9F | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  SWAP                  |                         |             |             |                        |       |   |   |       |

|  Swap Nibbles within   | A<7:4> -> A<3:0>        | Accumulator |             | SWAP  A                |  C4   | 1 | 1 | - - - |

|  the Accumulator       | A<3:0> -> A<7:4>        |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|                        |                         |  DEST       | SOURCE      |   ASSEMBLY             |HEX    |B  |C  | PSW   |

|   MNEMONIC             |  OPERATION              |  ADDR       |  ADDR       |   LANGUAGE             |OP-    |Y  |Y  |C A O  |

|                        |                         |  MODE       |  MODE       |     FORM               |CODE   |T  |C  |F C V  |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  XCH                   |                         |             |             |                        |       |   |   |       |

|  Exchange bytes of the | SOper<7:0> -> A<7:0>    | Accumulator | Direct      | XCH   A,Daddr          |  C5   | 2 | 1 | - - - |

|  Accumulator and the   | A<7:0> -> SOper<7:0>    |      "      | Indirect    | XCH   A,@Ri            | C6,C7 | 1 | 1 |       |

|  Source Operand        |                         |      "      | Register    | XCH   A,Rn             | C8-CF | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  XCHD                  |                         |             |             |                        |       |   |   |       |

|  Exchange the Least    | SOper<3:0> -> A<3:0>    | Accumulator | Indirect    | XCHD  A,@Ri            | D6,D7 | 1 | 1 | - - - |

|  Significant Nibble of | A<3:0> -> SOper<3:0>    |             |             |                        |       |   |   |       |

|  the Accumulator and   |                         |             |             |                        |       |   |   |       |

|  the Source Operand    |                         |             |             |                        |       |   |   |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

|  XRL                   |                         |             |             |                        |       |   |   |       |

|  Logical Exclusive OR  | SOper v DOper -> DOper  | Direct      | Accumulator | XRL   Daddr,A          |  62   | 2 | 1 | - - - |

|  of Source Operand     |                         |   "         | Immediate   | XRL   Daddr,#data      |  63   | 3 | 2 |       |

|  with Destination      |                         | Accumulator | Immediate   | XRL   A,#data          |  64   | 2 | 1 |       |

|  Operand               |                         |      "      | Direct      | XRL   A,Daddr          |  65   | 2 | 1 |       |

|                        |                         |      "      | Indirect    | XRL   A,@Ri            | 66,67 | 1 | 1 |       |

|                        |                         |      "      | Register    | XRL   A,Rn             | 68-6F | 1 | 1 |       |

|------------------------|-------------------------|-------------|-------------|------------------------|-------|---|---|-------|

BMULB           BYTE SIGNED MULTIPLY                                                                       JUL-30-84   PAGE 1

1    ;

2    ;  8-bit by 8-bit signed multiply--byte signed multiply

3    ;

4    ;    This routine takes the signed byte in multiplicand and

5    ;    multiplies it by the signed byte in multiplier and places

6    ;    the signed 16-bit product in product_high and product_low.

7    ;

8    ;    This routine assumes 2s complement representation of signed

9    ;    numbers.  The maximum numbers possible is then -128 and +127.

10    ;    Multiplying the possible maximum numbers together easily fits

11    ;    in a 16-bit product, so no overflow test is done on the answer.

12    ;

13    ;    Registers altered by routine: A, B, PSW.

14    ;

15    ;

16    ;  Primary controls

17    $MOD51

18    $TITLE(BYTE SIGNED MULTIPLY)

19    $DATE(JUL-30-84)

20    $PAGEWIDTH(132)

21    $OBJECT(B:BMULB.OBJ)

22    ;

23    ;

24    ;  Variable declarations

25    ;

00F0                26    sign_flag       BIT     0F0H            ;sign of product

0030                27    multiplier      DATA    030H            ;8-bit multiplier

0031                28    multiplicand    DATA    031H            ;8-bit multiplicand

0032                29    product_high    DATA    032H            ;high byte of 16-bit answer

0033                30    product_low     DATA    033H            ;low byte of answer

31    ;

32    ;

33    ;

0100                  34            ORG     100H                    ;arbitrary start

35    ;

0100                  36    byte_signed_multiply:

0100 C2F0             37                    CLR     sign_flag       ;reset sign

0102 E530             38                    MOV     A,multiplier    ;put multiplier in accumulator

0104 30E704           39                    JNB     ACC.7,positive  ;test sign bit of multiplier

0107 F4               40                    CPL     A               ;negative--complement and

BMULB           BYTE SIGNED MULTIPLY                                                                       JUL-30-84   PAGE 2

0108 04               41                    INC     A               ;add 1 to convert to positive

0109 D2F0             42                    SETB    sign_flag       ;and set sign flag

43    ;

010B 8531F0           44    positive:       MOV     B,multiplicand  ;put multiplicand in B register

010E 30F707           45                    JNB     B.7,multiply    ;test sign bit of multiplicand

0111 63F0FF           46                    XRL     B,#0FFh         ;negative--complement and

0114 05F0             47                    INC     B               ;add 1 to convert to positive

0116 B2F0             48                    CPL     sign_flag       ;complement sign flag

49    ;

0118 A4               50    multiply:       MUL     AB              ;do unsigned multiplication

51    ;

0119 30F00A           52    sign_test:      JNB     sign_flag,byte_signed_exit      ;if positive,done

011C 63F0FF           53                    XRL     B,#0FFh         ;else have to complement both

011F F4               54                    CPL     A               ;bytes of the product and inc

0120 2401             55                    ADD     A,#1            ;need add here because inc doesn't set

0122 5002             56                    JNC     byte_signed_exit        ;the carry flag

0124 05F0             57                    INC     B               ;if add overflowed A, inc the high byte

58    ;

0126                  59    byte_signed_exit:

0126 85F032           60                    MOV     product_high,B  ;save the answer

0129 F533             61                    MOV     product_low,A

62    ;

012B 22               63                    RET                     ;and return

64            END

ASSEMBLY COMPLETE, 0 ERRORS FOUND

BMULB           BYTE SIGNED MULTIPLY                                                                       JUL-30-84   PAGE 3

ACC                   D ADDR  00E0H  PREDEFINED

B                     D ADDR  00F0H  PREDEFINED

BYTE_SIGNED_EXIT      C ADDR  0126H

BYTE_SIGNED_MULTIPLY  C ADDR  0100H  NOT USED

MULTIPLICAND          D ADDR  0031H

MULTIPLIER            D ADDR  0030H

MULTIPLY              C ADDR  0118H

POSITIVE              C ADDR  010BH

PRODUCT_HIGH          D ADDR  0032H

PRODUCT_LOW           D ADDR  0033H

SIGN_FLAG             B ADDR  00F0H

SIGN_TEST             C ADDR  0119H  NOT USED